| Information Technology User Support Services |  |  |
| --- | --- | --- |
| Name: Accessibility Testing Product Instructions - 1194.23 Telecommunications Products |  |  |
| Prepared by: Jerry Garcia |  | Created on: 04/22/09 |
|  |  | Last update: 06/19/09 |
| Description: This document provides basic accessible functionality testing for Telecommunications Products. Additional testing might be required. |  |  |

Equipment, hardware and software to be used:

- TTY

- Phone

Instructions:

Use the following steps to test the product (ex. digital, analog, wire, wireless, and Internet-based products and devices such as PBX and telephone answering machines) for accessibility.  Please record all of your results and comments in the Accessibility Testing Product Results document.

- Does the product or system provide a function allowing voice communication? And does the product or system provide TTY functionality? If the answer is Yes to the first question, and No to the second question, then perform a visual inspection and/or check the manual for:

  - Check for a RJ-11 or a TSB-121 connection point.

  - Does product support 45.5 baud ASCII protocol?

  - Does product support 300 baud protocol ASCII?

- Is the product or system a voice mail, auto-attendant, or interactive voice response system?

  - Using TTY, leave a message.

  - Using TTY, navigate the automated menu if available.

  - If the product requires a voice response:

    - Is there a warning before the response time has elapse?

    - Does it provide sufficient time to indicate that more time is required?

    - Can response time be extended?

- Product or system provides Caller ID.

  - Does the product or service provides voice output or have a connection method for adding an external Caller ID product with voice output?

- Does the product or service transmit voice signals?

  - Does it have a volume control?

  - Check manual or test to find out if the volume can be amplified above the default level.

  - Check manual to find out if minimum volume is 20dB or greater.

- Products or Systems with option to adjust volume level.

  - Does volume resets back to default after every use?

- Does product have mechanically operated controls or keys?

  - If a keyboard is used, does it have navigational bumps?

  - Perform the basic functions with one hand.

  - Do the controls or keys required tight grasping, pinching or twisting of wrist.

  - If product has key repeat function, change the repeat rate.

  - Lock or toggle any controls/keys.

  - Do locking or toggle controls/keys have either an audible or tactile indicator for status?

---