| Information Technology User Support Services |  |  |
| --- | --- | --- |
| Name: Accessibility Testing Product Instructions - 1194.25 Self-Contained, Closed Products |  |  |
| Prepared by: Jerry Garcia |  | Created on: 04/23/09 |
|  |  | Last update: 06/11/09 |
| Description: This document provides basic accessible functionality testing for Self-Contained, Closed Products. Additional testing might be required. |  |  |

Equipment, hardware and software to be used:

- Wheelchair or chair.

Instructions:

Use the following steps to test Self-Contained, Closed products (ex. copiers, printers, fax machines, calculators, and information kiosks) for accessibility.   Please record all of your results and comments in the Accessibility Testing Product Results document.

- If controls or keys are used:

  - Touch and feel the controls or keys. Are controls/keys large and tactilely discernible?

  - Perform basic functions with one hand.

  - If product has key repeat function, change the repeat rate.

  - Lock or toggle any controls or keys.

  - If touch-screen or contact sensitive controls are used, can you select or activate any of the functions or controls other than using your fingers?

  - Do locking or toggle controls/keys have either an audible or tactile indicator for status?

  - Are buttons label?

- If auditory information or output is used:

  - Can you find and see the same information anywhere?

  - Is there a standard audio connection point?

  - Are there controls for audio interrupt, pause, and restart?

  - Does the volume automatically reset to the default level after every use?

- If visual information is used, can you hear the same information?

- If color is used to convey information:

  - Is there another way the same information is being provided?

- If color and contrast adjustments are available, change settings to full-color and monochrome.

- If product has a visual display screen, verify that the flicker on the screen is nonexistent or very slow.

- If the product is freestanding, non-portable and intended to be used in one location and which has operable control:

  - Sit in front of the product in a wheelchair or in a chair with a seat height of approximately 23.75 inches.

  - Verify that all controls can be reach and operated easily from the seating position.

---